High Level Changes in eMarket

As part of the Electric Storage Devices (ESD) project, eMarket will be enhanced to allow for Continuous Storage Facilities (CSFs), a new type of resource, capable of automatically switching between generation and consumption and provide regulation. In the eMarket application these facilities are referred to as Electric Storage Devices (ESDs).

There are no changes in the eMarket User Interface or Web Services for non-CSF assets.

A CSF is created by registering three asset types in the Customer and Asset Management System (CAMS) as part of a single facility, each with separate asset IDs and names:

CSF Generator

CSF Dispatchable Asset-Related Demand (DARD)

CSF Alternative Technology Regulating Resource (ATRR)

All three components of a CSF may participate simultaneously in its applicable market:

- Generators manage energy and reserves for MW at and above zero

- DARDs manage energy and reserves for MW below zero (consumption)

- ATRRs manage regulation, positive or negative

The components of a CSF will be managed independently using the same eMarket group functions as non-CSF assets, i.e. the CSF generators are managed using the generation tab, DARDs using the ARD tab, and ATRRs using the regulation tab. There is no cross-validation between generation and consumption parameters of assets belonging to the same CSF.

The eMarket portfolio functionality will be the same for CSF and non-CSF assets.

Both the CSF Generator and the CSF DARD will always be online in the Day-Ahead Market, unless the participant declares them unavailable.

eMarket User Documentation

The following documents were updated for ESD changes and will be available on the ISO-NE website:

eMarket User Guide v.6.0 – user interface guide containing detailed information about CSF concepts and validation rules

eMarket Data Exchange Specification v.10.7 – web services interface guide